- The University of Western Australia

School of Engineering

Prof. Thomas Bräunl

- Robot Manipulators and Automation

AUTO4507

Lab Assignment 1 –Group– Industrial Automation	Points: 5+5 = 10

Pre-Lab – Complete before the lab session

Before the first lab, answer pre-lab questions by reviewing the documentation in the Festo directory: 
http://roblab.org/courses/mobrob/project/festo/LabA-Festo/PrelabDocumentation/

| All stations: - What is the operating pressure of each station? - What is the operating voltage of each station? - A compressor can generate 28l/min of air for all 5 Festo modules, is this sufficient? - What is the comparative height difference between the red, silver, and black workpieces? |  |
| --- | --- |
| Through Beam Sensor Receiver: SOEG 1. What is the maximum range? 2. What is the measurement principle? |  |
| Sorting station – Sensors: 1. (Proximity-SIEN): a. Operating Principle? b. Max switching distance? 2. (Proximity-SME): a. Operating Principle? 3. (Retro-Reflective-SOEG): a. Operating Principle? b. Coverage Range? 4. (Transmitter-SOEG): a. Operating Principle? b. Coverage Range? |  |
| Pick and Place station – Vacuum Generator: - A module supplies 1100kPa of pressure, can it be connected to this unit? - A workpiece requires -3 psi to be lifted, why can/can’t this module be used? |  |

Experiment 1 – Codesys

- Install and use CODESYS to determine, from the global variable lists, the PLC addresses (e.g., %IX1.0) corresponding to the variables listed in the tables below.

- Indicate whether the variable is an input (e.g., provides a reading from a sensor) or an output (e.g., provides a signal to trigger a motor)

- Provide a short description for the meaning of a high value corresponding to each variable (e.g., for an output variable: when high, conveyor moves forward; or, for an input variable: when high, workpiece is at sensor)

Variable Matching:

- Feeder Station:

| Variable | PLC Address |  | Input/ Output? |  | Meaning of high input/output |  |
| --- | --- | --- | --- | --- | --- | --- |
| FB_Mod_SM |  |  |  |  |  |  |
| xSlideRetracted |  |  |  |  |  |  |
| xSlideExtended |  |  |  |  |  |  |
| xMagEmpty |  |  |  |  |  |  |
| xExtendSlide |  |  |  |  |  |  |
| FB_SeqNTransport_DI |  |  |  |  |  |  |
| xSensorStart |  |  |  |  |  |  |
| xSensorMid |  |  |  |  |  |  |
| xSensorEnd |  |  |  |  |  |  |
| xConvForward |  |  |  |  |  |  |
| xConvBackward |  |  |  |  |  |  |

- Measuring Station:

| Variable | PLC Address | Input/ Output? | Meaning of high input/output |
| --- | --- | --- | --- |
| FB_Mod_ME |  |  |  |
| xGripperOpened |  |  |  |
| xGripperIsUp |  |  |  |
| xSwivelDriveLeftPos |  |  |  |
| xSwivelDriveRightPos |  |  |  |
| xOpenGripper |  |  |  |
| xGripperDown |  |  |  |
| xSwivelDriveRotateClockwise |  |  |  |
| FB_SeqNTransport_ME |  |  |  |
| xSensorStart |  |  |  |
| xSensorMid |  |  |  |
| xSensorEnd |  |  |  |
| xConvForward |  |  |  |
| xConvBackward |  |  |  |
| xExtendSeperator |  |  |  |
| xRetractStopper |  |  |  |

- Pick and Place Station:

| Variable | PLC Address | Input/ Output? | Meaning of high input/output |
| --- | --- | --- | --- |
| FB_Mod_PP |  |  |  |
| xSlideRetracted |  |  |  |
| xSlideExtended |  |  |  |
| xSuctionCupUp |  |  |  |
| xWorkpiecePieckedUp |  |  |  |
| xRetractSlide |  |  |  |
| xExtendSlide |  |  |  |
| xSuctionCupDownwards |  |  |  |
| xVacuumOn |  |  |  |
| FB_SeqNTransport_PP |  |  |  |
| xSensorStart |  |  |  |
| xSensorMid |  |  |  |
| xSensorEnd |  |  |  |
| xConvForward |  |  |  |
| xConvBackward |  |  |  |
| xExtendSeparator |  |  |  |

- Pressure Station:

| Variable | PLC Address | Input/ Output? | Meaning of high input/output |
| --- | --- | --- | --- |
| FB_Mod_RE |  |  |  |
| xForkLightBarrier |  |  |  |
| xReflexLightSensor |  |  |  |
| xInductiveProximitySensor |  |  |  |
| FB_SeqNTransport_SO |  |  |  |
| xSensorStart |  |  |  |
| xConvForward |  |  |  |
| xSeparator1Exdented |  |  |  |
| xSeparator2Exdented |  |  |  |
| xSensorSlides |  |  |  |
| xExtendSeparator1 |  |  |  |
| xExtendSeparator2 |  |  |  |
| xRetractStopper |  |  |  |

- Sorting Station:

| Variable | PLC Address | Input/ Output? | Meaning of high input/output |
| --- | --- | --- | --- |
| FB_Mod_RE |  |  |  |
| xForkLightBarrier |  |  |  |
| xReflexLightSensor |  |  |  |
| xInductiveProximitySensor |  |  |  |
| FB_SeqNTransport_SO |  |  |  |
| xSensorStart |  |  |  |
| xConvForward |  |  |  |
| xSeparator1Exdented |  |  |  |
| xSeparator2Exdented |  |  |  |
| xSensorSlides |  |  |  |
| xExtendSeparator1 |  |  |  |
| xExtendSeparator2 |  |  |  |
| xRetractStopper |  |  |  |

# Experiment 2 – Run the Automation System

During the second lab each group will be assigned a module of the Festo production line.

Tasks

- In the provided code for each station, several variables have been replaced with a variable called ‘Dummy_Var’. Replace all dummy variables with the correct variable (provided in the tables above) o Note: if a dummy variable corresponds to an action, the action type (S, R, N) may need to be changed too.

- As a group, demonstrate uploading the working program to the Festo station and running your program.