- The University of Western Australia

School of Engineering

Prof. Thomas Bräunl

- Robot Manipulators and Automation

AUTO4507

Lab Assignment 3  –Group– Serial Robot  Points: (1+2+2)+(3+2) = 10

Make yourself familiar with the UR5 robot documentation and the interfacing via Ethernet.

Experiment 1 – Moving in Space

1. 1 Square

- Joint Space

Write a program that moves the robot in a sequence to the 4 corner points of a square of size 20cm. Let the robot stop for 1 second after reaching each position.

- Cartesian Space
Repeat the same experiment, but let the arm move along a straight line

1. 2 Cube

- Joint Space

Write a program that moves the robot in a sequence to the 8 corner points of a cube of size 20cm. Let the robot stop for 1 second after reaching each position.

- Cartesian Space
Repeat the same experiment, but let the arm move along a straight line

1. 3 Circle

Write a program that moves the robot gripper along a circle of diameter 20cm.

Experiment 2 – Vision System

Let the robot arm rest about 50cm above the table with the camera looking down.

Using OpenCV in Python or C++, solve the following tasks.

2.1 Classification

Identify all objects on the table and classify them as either triangle, square, (half-) circle or star shapes using OpenCV function:  approxPolyDP
See link for shape detection:

https://stackoverflow.com/questions/11424002/how-to-detect-simple-geometric-shapes-using-opencv

Print out each object’s type.

2.2 Calculation

For each detected object:

- Calculate and print the global coordinates of its 2D center-point.

- Move the robot arm so the object’s center is in the center of the camera image.