INFORMATIONS TO ROBOSIM

This program has been generated for a software practical course by Johannes

Sch�tzner,under the direction of Dr.Thomas Br�unl, University of Stuttgart,

in 1996. It is based on the 'Studienarbeit' by Rainer Pollak, made in 1992

and programmed in C.

->Introduction

RoboSim is a program which simulates a six-link robot. You can control the

robot in several ways and manipulate it, for example giving the links new

lengths or changing the weights belonging to the indicated angles. It's also

possible to make the robot save pre-allocated positions and to teach the robot

a sequence of movements.Also, this software can be used to control a real

robot. This robot is a Lynxmotion manipulator, available from Joker Robotics.

In the following passage the possible commands are explained in more detail.

->Useful Advice

This Java applet needs a reserved place of 920x670 pixels. Otherwise parts of

the drawing are hidden by the panel on the right side. If you can't see a ro-

bot between the control panels, it's useful to click several times between the

panels. If the robot seems to stop while doing a move and the scrollbar labels

disappear, it might be helpful to move the mouse across the robot.

->Restrictions

Since there exist a lot of security restrictions, the loading and producing of

control programs can only be done with the Java interpreter. Unfortunately,

applets started with Netscape are not allowed to do this.

The real robot can only be controlled by a special RoboSim version, because

its control needs C-code which has to be integrated into this Java program.

This integration would make it impossible for this program to run on every

computer system. So the integration has been realized only on a limited

version.

->The Commands

Forward Kinematics:

This command is used to control the robot and its rotation links direct-

ly by modifying the angles corresponding to these links. The angles' values

are restricted to sensible ranges which are shown above the scollbars. They

are as follows:

Angle 1: 180.0 degree to -180.0 degree

Angle 2: 30.0 degree to -210.0 degree

Angle 3: 30.0 degree to -210.0 degree

Angle 4: 180.0 degree to -180.0 degree

Angle 5: 90.0 degree to -90.0 degree

Angle 6: 180.0 degree to -180.0 degree

The real robot has not got the facilities of angles 4 and 6, so these angles

are not available to the user.

Inverse Kinematics:

You can control the robot by giving it a target position (x-, y-, and z-value)

with this command. The movement is executed after a click on the execute, bu-

tton just below the scrollbars of the inverse kinematics. If a target point is un-

reachable, the robot stops. By pushing on the other button in the auto control

area, you can select, whether the robot should move in a straight line or on an

arbitrary trajectory.

Determination of the arm lengths:

This command allows you to vary the length of the arms of the robot. Also

there exists a test for collision. If a collision is detected, the change of the

arm lengths is not executed. The execution is only performed, if the corresponding

execute button is pushed. The possible new lengths are limited to sensible ranges

shown above each scrollbar. They are as follows:

Arm 1: 15.0 to 4.5

Arm 2: 15.0 to 2.5

Arm 3: 15.0 to 2.5

If the real robot is controlled, this command can't be selected, because the

real robot has fixed arm lengths.

Angle weights:

Again, the values given to the text fields are only accepted after pushing

the execute button. Then you have new angle weights. They are needed to obtain the

best solution of the inverse kinematics. The values have to be positve integers.

Point of view:

You can change the point of view by dragging the robot with the mouse in any

direction. By using the zoom scrollbar, you are able to increase and decrease the

size of the robot.

Speed:

You can select your required speed with the corresponding scrollbar. By

switching off the double buffering mode, the robot motion is much faster.

Define/Go to positions:

Three positions can be defined simply by pushing the define pos button of your

choice. By doing this, the six actual angles of the robot and the actual open/

close state of the hand are saved. This position can be restored by pushing the

equivalent go to button. The first button (Go to Init ) holds the angles of the

robot at the beginning. If you want to see an example movement, push the example

button.

Virtual/Real robot:

You can choose between the virtual robot as shown in the middle of your screen

and a real robot (Lynxmotion manipulator). Selecting the real robot, the arm

lengths and the angles four and six can't be changed, since the real one consists

of only four links.

Open File:

By selecting this button, you can load a file which either has been written

"by hand" or has been produced with the Teach button. How to write executable

files and use the Teach button, see the following links.

Teach:

After having activated this button, a file dialog window appears in which you

can choose the name and the directory of the file which will be produced and

written automatically. By doing this, the Teach button will be replaced by the

Enter button and the Stop button will be enabled. Now you can control the ro-

bot as usual, but every time you push the Enter button, the actual robot

angles and the actual open/close state of the hand are written into the indicated file.

This mode is only stopped, if the Stop button is pushed. At the beginning of

this mode, the actual linklengths,

speed, and zooming factor are stored.

Writing own control programs:

You can write control programs such like these ones being produced by the Teach

button, but you have got more command power. Every command starts with a com-

mand character and is completed by the arguments needed for this command.

Every input, e.g. command character and arguments, must be in a seperate line.

Otherwise the required motion of RoboSim cannot be guaranteed. The possible

commands are:

"f" This character stands for forward kinematics. It has to be followed by six

numbers in double format representing the new angles. Remember that every command

and every number has to be in a row of its own.

"i" indicates inverse kinematics. In the next three lines there must be the three

coordinates of the target point.

"l" By choosing this character, which represents linklengths, you can deter-

mine new lengths for the links. These lengths has to be in the allowed ranges.

"w" is the abbreviated form for angle weights . The arguments of this command

are the six new angle weightswich are integers.

"s" represents speed. It has only one argument - the new speed. The range of

possible values starts at 1 (slow) and goes up to 82 (fast).

"h" signifies that the content of the following line should be regarded as the

open/close state of the robot's hand. 0.0 means a fully open hand and 1.0 a

totally closed one.

"z" indicates the zooming factor. The zooming factor range is between 0.1

(very tiny) and 3.5 (large).

"e" must be the last command of every control program and means end.

"#" Comments start with # and last till the end of line.

The system is not case sensitive.

Example:

# Automatic generated file (Mon Jul 08 12:43:30 MET DST 1996)

l

13.5

10

10

s

25

z

1.5

f

0

0

0

0

0

0

f

0

-33.1397

23.7784

0

9.36132

0

f

-43.0761

-26.211

-1.94342

0

28.1545

-43.0761

f

-51.3402

-18.5244

-22.2129

0

40.7374

-51.3402

f

0

0

0

0

0

0

e